Model Questions of Electrical Equipments in Mines

- Classify cables for mining use .

- Constructional features of high tension and low-tension cables armored & trailing cables.

- State size of cables & their use.

- State procedures of cable laying at surface, underground roadway & in shafts.

- Describe cable joint box mining type.

- What is a fuse ? Discuss the advantages and disadvantages of a fuse.

- Why do we prefer silver as a fuse element ?

- Define and explain the following terms :

- (i) fusing current (ii) cut off current (iii) operating time (iv) breaking capacity

- Write short notes on the following :

- (i) Semi-enclosed rewireable fuse

- (ii) H.R.C. cartridge fuse

- (iii) Difference between a fuse and circuit breaker

- What is a circuit breaker ? Describe its operating principle.

- Discuss the arc phenomenon in a circuit breaker.

- Explain the various methods of arc extinction in a circuit breaker.

- Define and explain the following terms as applied to circuit breakers :

- (i) Arc voltage

- (ii) Restriking voltage

- (iii) Recovery voltage

- Describe briefly the action of an oil circuit breaker. How does oil help in arc

extinction ?

- Discuss the advantages and disadvantages of oil circuit breakers.

- Explain with neat sketches the construction and working of the following

circuit breakers:

- (i) Plain explosion pot

- (ii) Cross jet explosion pot

- (iii) Self-compensated explosion pot

- Explain the difference between bulk oil circuit breakers and low-oil circuit

breakers.

- Discuss the constructional details and operation of a typical low-oil circuit

breaker? What are its merits and demerits?

- Discuss the principle of operation of an air-blast circuit breaker. What are

the advantages and disadvantages

- Explain briefly the following types of air-blast circuit breakers Axial-blast

type, Cross-blast type

- What are the important components common to most of circuit breakers?

Discuss each component briefly.

- Write a short note on the rate of re-striking voltage indicating its

importance in the arc extinction.

- Discuss the phenomenon of Current chopping and Capacitive current

breaking.

- Write short notes on the following -Resistance switching, Circuit breaker

ratings, Circuit interruption problem

- Explain essential qualities of a good protective system.

- Explain with the help of neat diagram the construction and working of :

- Non-directional induction type overcurrent relay

- Induction type directional power relay

- Describe protection of transformer by differential relay.

- Describe general principle of working-basis remote control circuit &

various protective devices of Gate-End Box

- Describe functions & operation of drill panel.

- Earthing system in mines.

- Working principle of transformer

- Explain starting & running characteristics of D.C motors

- Explain starting & running characteristics of A.C motors

- State selection of motors for mining use.

- Describe & explain regenerative braking.

- Define flame proof apparatus & intrinsically safe apparatus.

- Describe & explain the safety features of flame proof & intrinsically safe

apparatus.

- Describe & explain magnetic braking.

- Describe signals & shaft signal.

- Describe communication system in U/G mines.

- Explain Point to point communication.

- Explain Intercom system/Telephone

- Explain Cordless system.

- Describe Sensors & their applications.

- Write short notes on -Battery locomotive, Automation with Thyroster

control, Electrical LHD, Electric mine phone.